McGowan Lake Campground Inc.

Holding Tank Specifications – Schedule C

Installing and setting up a new drum:

- Must be no more than 45 gallons

- Completely sealed plastic drum

- Trailer to drain into drum through 3” ABS completely exposed piping sealed with glue and/or calking to drum exterior **Flex hose is only to be used where ABS cannot and must be approved by management**

- Outtake to be fitted with 3” ABS flange and sealed with flexible silicone

- Drum must be fitted with a separate intake and outtake cleanout

- Top of drum to be exposed 2” above ground to allow viewing access to top of drum exterior

**Please see management for best possible location of drum for accessibility of honey wagon and to avoid water and hydro line damage. Remember CALL BEFORE YOU DIG**

Regular Weekly Maintenance and Upkeep to be performed by the Occupant:

- All septic areas must be clean, easily accessible and free of debris including leaves, grass, weeds, rocks and dirt buildup

- Before pulling your valves, make sure that your glue and/or calking has not come away from the intake/outtake seals on the drum

- When pulling your valves, check at all connections to make sure there is nothing leaking

- Make sure that the cap is easily removable for clean outs

- Septic treatment must be used at the beginning of the season and after every pumpout

Pumpouts will not be done if:

- The area around the septic is not maintained

- There are any leaks

- Management has asked you to repair an issue and it has not been resolved

**ALL grey water and black water MUST go into the septic drum**

Initials: __________